| HTA Reg 413/05 (Section being amended) | Proposed Amendment | Explanation of Change |
| --- | --- | --- |
| s3 (Schedules 1-13) | Wide Single Truck Tires In each schedules’ Weight Limit Chart: a) Clarify that weight limits for “individual axles with single tires” does not include steering axles, and b) Extend upper limit of table to 9000 kg rather than current 8000 kg. | - Phase 3 of vehicle weight and dimension reforms was intended to introduce Safe, Productive, Infrastructure-Friendly (SPIF) vehicles which better harmonize with other jurisdictions. - This amendment removes an interim 8000 kg weight restriction which was applied to non-steering axles of SPIF vehicles equipped with wide single tires pending further pavement research. That research is complete and MTO has a better understanding of the impact of these tires on pavement. - Allowing weights up to 9000 kg harmonizes with allowances in Quebec. |
| s3 (Schedules 1-13) | Single Tire Track Width In each schedules’ Dimensional Limit Chart: a) Add an exception to all references to “Track Width”. Instead of the specified dimension, vehicles equipped with single tires and built prior to 2010 are allowed a track width of 2.3 to 2.6m. | - Phase 3 of vehicle weight and dimension reforms was intended to introduce Safe, Productive, Infrastructure-Friendly (SPIF) vehicles which better harmonize with other jurisdictions. - This amendment relaxes the track width requirements for SPIF vehicles equipped with single tires built prior to 2010. - Single tires must be mounted on longer axles than dual tires to satisfy the track width requirements. With the advent of wide single tires and removal of regulatory impediments, operators may wish to retrofit existing trailers from dual to single tires. - This change is consistent with harmonization obligations contained in a memorandum of understanding signed by all provinces. |
| s3 (Schedule 6) | Michigan Interaxle Spacing In Schedule 6 Dimensional Limit Chart: a) Amend dimension (17) “Interaxle Spacing” to 2.3 to 2.5m. | - Phase 3 of vehicle weight and dimension reforms was intended to introduce Safe, Productive, Infrastructure-Friendly (SPIF) vehicles which better harmonize with other jurisdictions. - This amendment allows slightly more space between the rear axles of Designated Combination 6 to accommodate an additional axle for use in Michigan. - This change improves harmonization of weight and dimension rules with Michigan. |
| s3 (Schedules 1-3) | Grandfather Provisions In Schedule 1 Dimensional Limit Chart: a) Add an exemption to dimensions (11), (12), (13) and (19) that, until December 31, 2019, these dimensional limits do not apply to semi-trailers 14.65m or less in length built prior to 2006. In Schedules 2 and 3 Dimensional Limit Charts: a) Add an exemption to dimensions (11) and (19) that, until December 31, 2019, these dimensional limits do not apply to semi-trailers 14.65m or less in length built prior to 2006. | - Phase 3 of vehicle weight and dimension reforms was intended to introduce Safe, Productive, Infrastructure-Friendly (SPIF) vehicles which better harmonize with other jurisdictions. As many of the standards apply only to new vehicles, grandfather protection was extended to existing vehicles. - This amendment corrects an oversight in those grandfather provisions and allows an exemption from certain dimensional limits for semi-trailers 14.65m or less in length built prior to 2006. |
| s32(1)(2) | Double Trailer Weight Reductions In each subsection, following the words “that is not a designated combination 11 or 13” and “or is a double trailer combination that is not an A-, B- or C-train double”. | - Phase 3 of vehicle weight and dimension reforms was intended to introduce Safe, Productive, Infrastructure-Friendly (SPIF) vehicles which better harmonize with other jurisdictions. Weight penalties are applied to vehicles that do not meet the stated standards. - This amendment removes a potential ‘loophole’ in applying weight reductions to undesirable double trailer combinations. |
| New | Fire Truck Weight Exemption - Exempt fire trucks from HTA axle and gross weight limits providing the vehicle and its components do not exceed manufacturers’ rated capacities. | - Modern fire trucks often exceed HTA axle and gross weight limits when fully loaded with emergency equipment and water. This amendment provides an exemption from HTA weight limits, providing the trucks do not exceed manufacturers’ rated capacities. |