| Topic | Details | Rationale | Vehicle |
| --- | --- | --- | --- |
| Cab | Add the words “cab or sleeper” to the major defect list as follows: “Any cab or sleeper door fails to close securely” | - Match federal standard, National Safety Code standard 13, Trip Inspection (NSC 13) | Truck |
| Cargo Securement | Add the example to the minor defect as follows: “Insecure or improper load covering (e.g. wrong type or flapping in the wind)” | - Match NSC 13 | Truck Trailer Bus |
| Suspension System | Add clarification to “damaged” by adding the following: “Damaged = patched, cut, bruised, cracked to braid, mounted insecurely”. | - Match NSC 13, greater clarity | Truck Trailer Bus Motor Coach |
| Tires | Both Regulation 199/07 and NSC 13 identify tires leaking as a minor defect and a flat tire as a major defect. Proposed amendment to identify if the leak can be felt or heard it is a major defect. | - Match NSC 13 | Truck Trailer Bus Motor Coach |
| Brake Chamber | Remove the requirement “DD3 brake chamber fails to hold vehicle during tug test, when all air reservoirs are drained”. | - Match NSC 13, safety concerns | Motor Coach |
| Hydraulic Brake System | An inoperative parking brake is a minor defect in Regulation 199. Proposed amendment to change to a major defect. | - Match NSC 13, was previously an error in the regulation | Truck Trailer Bus |
| School Bus | Proposed amendment to allow an operator to drive a school bus with inoperative overhead warning lights or stop arm when these devices are not required. | - Allows the vehicle to be driven to a place of repair. - Allows the vehicle to be move to a safe location to stop and transfer children i.e. parking lot. - No negative impact to safety as the vehicle is still prohibited to be used when these features are required. | Bus |
| Electronic Inspection Reports | Proposed amendment to require drivers using electronic inspection reports to provide a printed, emailed, faxed or handwritten report based on the information in the electronic device upon the request of a police officer or inspection officer. |  | Truck Bus |
| Definitions | Proposed minor wording changes to definitions to match recent amendments in other Highway Traffic Act regulations. - “House Trailer” to match Regulation 611, Safety Inspections, - “Motor Coach” change the term to “intercity bus” and update the definition to match Regulation 413/05, Vehicle Weights and Dimensions – for Safe, Productive and Infrastructure-Friendly Vehicles. - “School Purposes Vehicle” to match Regulation 611, Safety Inspections. | - Add clarity and consistency in the terms used throughout Ontario legislation and regulations. | N/A |