Proposed draft regulatory language for the amending of Regulation 629, Vehicles For The Transportation Of Physically Disabled Passengers

Please note:  This is a proposed draft only and is subject to change.

Highway Traffic Act

ACCESSIBLE VEHICLES

1.  In this Regulation,

“accessible taxicab” means an accessible vehicle that is licensed as a taxicab by a municipality

“accessible vehicle” means,

a passenger vehicle or a bus designed or modified to be used for the purpose of transporting persons with disabilities and used for that purpose, whether or not the vehicle is also used to transport persons without disabilities, if the vehicle is operated

- for compensation, or

(ii) 	not for compensation by, for or on behalf of any person, club, agency or organization that holds itself out as providing a transportation service to persons with a disability;

2. (1)  For the purposes of section 102(3) of the Act, the standards set out in the Canadian Standards Association Standard D409-M84 are prescribed for accessible vehicles manufactured on or after January 1,1986

(2)  Equipment conforming with the Canadian Standards Association Standard D409-M84 is prescribed and required for use in all vehicles of the class referred to in subsection (1).

3.  (1)  Every accessible vehicle shall,

(a) be equipped with an interior mirror designed to provide the driver with a view of the passengers;

(b) be equipped with lights arranged to illuminate all of the interior of the vehicle that shall be constantly lit during the loading or unloading of passengers and that shall be sufficient to permit the safe on-board movement of passengers;

(c) be equipped with lights above or beside each passenger access door that are constantly lit when the door is open and that,

(i) illuminate the ramp, lift or step nosings, and

(ii) in the case of vehicles manufactured after 2012, illuminate the ground surface for a distance of at least 0.9 m perpendicular to the outer edge of a lift or the bottom step tread.

(c.1) the lights required by clause (c) shall be shielded to protect the eyes of entering and exiting passengers;

(d) be equipped with a means of securing in the open position, each passenger access door or emergency exit door that could be subject to unintentional closing during the loading or unloading of passengers;

(d.1) in the case of vehicles manufactured before 2013 with grab bars, handholds, handrails or stanchions,, have all such devices meet the following requirements:

(i) they are securely mounted,

(ii) they are colour contrasted,

(iii) they have a slip resistant surface

(iv) they have an exterior diameter that permits easy grasping, and

(v) they have sufficient clearance from the surface to which they are attached to permit easy grasping.

(d.2) in the case of vehicles manufactured after 2012, be equipped with grab bars, handholds, handrails or stanchions that meet the following requirements:

(i) they are securely mounted,

(ii) they are colour contrasted,

(iii) they have a slip resistant surface,

(iv) they have an exterior diameter that permits easy grasping,

(v) they have sufficient clearance from the surface to which they are attached to permit easy grasping,

(vi) they are located at each entrance, including any wheelchair entrance, and exit, are accessible from the ground, and are mounted such that they are inside the vehicle when the doors are closed,

(vii) they are located  where passengers are required to pay fares, and

(viii) they are distributed within the vehicle to facilitate the safe on-board movement of passengers as appropriate to the vehicle’s design.

(e) have an interior that is free of any sharp projections that may constitute a hazard to passengers;

(f) have a floor in the aisle and steps that are slip resistant, fire-retardant, securely bonded or fastened and that produces minimal glare composed of either

(i)  wear-resistant rubber or equivalent material, or

(ii) carpet with a low, firm and level pile or loop.

(g)  have steps that,

(i) have full-width yellow or white step nosings that are contrasted to their background,

(ii) have closed risers, and

(iii) have uniform riser heights and tread depths, subject to the structural limitations of the vehicle.

(h) conform with Canada Motor Vehicle Safety Standard 302 regarding flammability;

(i) be equipped with at least one dry-chemical-type fire extinguisher,

(i) bearing the label of a recognized testing agency,

(ii) showing a rating of not less than 2-A:10-B:C, and

(iii) equipped with a pressure gauge indicating that the fire extinguisher is adequately charged,

contained in the extinguisher manufacturer’s bracket; and

(j) be equipped with a unitized first aid kit in a sturdy, dustproof removable container containing,

(i) packets containing hand cleansers and gauze cleansing pads,

(ii) adhesive dressings, individually wrapped,

(iii) compress dressings,

(iv) eye dressing kits,

(v) gauze dressings,

(vi) gauze bandages,

(vii) adhesive tape,

(viii) triangular bandages,

(ix) rolled metal splint,

(x) one pair of scissors,

(xi) one pair of sliver tweezers, and

(xii) safety pins.

(2)  The fire extinguisher and first aid kit required by subsection (1) shall be mounted or secured in a location readily accessible to the driver and, if not in plain view, the location thereof shall be plainly marked.

3.1. (1) Every accessible vehicle shall have at least one door for passenger access and an emergency exit door, located on different walls of the vehicle, and the emergency exit door shall be operable from both inside and outside the vehicle.

(2)  In the case of an accessible vehicle used for the transportation of persons in wheelchairs, each of the doors referred to in subsection (1) shall have a wheelchair passageway not less than 762 millimetres wide.

(3) Subsection (2) and the door opening requirements in section 6.9.1 of the Canadian Standards Association Standard D409-M84 do not apply to a vehicle with a gross vehicle weight rating of 2,900 kilograms or less that is equipped with two or fewer wheelchair securement devices.

(4)  A vehicle to which subsection (3) applies shall have,

(a) a door for passenger access with a clear opening of not less than 762 millimetres wide; and

(b) an emergency exit door with a clear opening of not less than 724 millimetres wide.

4. (1)  Every accessible vehicle used for the transportation of persons in wheelchairs shall display the international symbol of access in a clearly visible position on the rear of the vehicle and on the front of the vehicle in a position other than on the windshield.

(2)  The symbol of access required by subsection (1) shall,

(a) have a height of not less than fifteen centimetres and a width of not less than fifteen centimetres; and

(b) consist of a symbol in white on a blue background.

5.   (1)  Every loading ramp used on a accessible vehicle shall,

(a) have a slip resistant surface;

(b) have raised edges of sufficient height to prevent a wheelchair from rolling off the edge of the ramp during the loading or unloading of passengers;

(c) be secured by means other than a support or lug in the door while the vehicle is operated on a highway. and

(d) have a coloured contrasted strip running its full width marking the bottom of the ramp.

(2)  Every power lift used on an accessible vehicle shall,

(a) have a slip resistant platform surface;

(b) have the sides of the platform fitted with raised edges extending the full length on each side and of sufficient height to prevent a wheelchair from rolling off the platform during vertical operation of the lift;

(c) have a retractable lip on the outer edge of the platform of sufficient height to prevent a wheelchair from rolling off the platform during vertical operation of the lift;

(d) be secured by means other than a support or lug in the door while the vehicle is operated on a highway; and

(e) be capable of raising and lowering a minimum weight of 275 kilograms, and

(f) have a coloured contrasted strip running its full width marking the edge of the platform at end at which the wheelchair enters from the ground.

(3) Every accessible vehicle manufactured after 2012 that is equipped with a power operated loading ramp or power lift or that has a kneeling feature shall also be equipped with both a warning lamp indicator mounted on its exterior near its entrance and an audible warning alarm.

(4) The indicator and alarm required by subsection (3) shall function while,

(a) the power operated ramp is being extended or retracted

(b) the power lift is being raised or lowered, or

(c) a vehicle that can kneel is being lowered or raised.

(5) Subsections 5(3) and (4) do not apply to a vehicle if the vehicle’s permit was issued by a jurisdiction other than Ontario.

5.1 (1) Every designated wheelchair position in an accessible vehicle manufactured after 2012 shall,

(a) in the case of a school bus or a vehicle with a designated seating capacity of not more than 24 passengers, have an allocated space of 685 mm from side to side of the vehicle and 1220 mm from front to back of vehicle, and

(b) in the case of a vehicle that is not a school bus and that has a designated seating capacity of more than 24 passengers, have an allocated space of 760 mm from side to side of the vehicle and 1220 mm from front to back of vehicle.

(2) A vehicle to which subsection (1) applies may have an overlap between designated wheelchair positions of not more than 150 mm from front to back of vehicle.

6.  (1)  Every accessible vehicle used for the transportation of persons in wheelchairs shall be equipped with wheelchair securement devices that limit the forward, backward, rotational, lateral and vertical motion of every wheelchair in the vehicle at the points of contact of the wheelchair with the vehicle while the vehicle is in normal operation.

(2)  For the purpose of subsection (1), normal operation includes full throttle acceleration and maximum braking and cornering.

(3)  Every wheelchair securement device in an accessible vehicle shall be,

(a) securely anchored to the vehicle; and

(b) capable of withstanding a load in any direction of not less than 11120 newtons for a period of ten seconds..

(4)  Where more than one wheelchair securement device is used to secure a wheelchair in an accessible vehicle, clause (3) (b) applies to the combination of devices used and not to each single device.

7.  (1)  Every accessible vehicle shall be equipped with a pelvic-type occupant restraint assembly, securely anchored to the vehicle, for each seat in the vehicle used for the carriage of persons with disabilities.

(2)  Every designated wheelchair position in an accessible vehicle shall, in addition to the devices required by section 6, be equipped with an occupant restraint assembly,

(a) securely anchored to the vehicle; or

(b) capable of securing a wheelchair occupant to the wheelchair by encircling both the occupant and wheelchair. s. 7 (2).

(3)  Every occupant restraint assembly in an accessible vehicle shall be capable of withstanding a load in any direction of not less than 11120 newtons for a period of ten seconds.

8.    (1)  For the purposes of section 102(3) of the Act, the following standards are prescribed for rear-entry accessible taxi-cabs manufactured on or after January 1, 1986:

- the standards set out in the Canadian Standards Association Standard D409-M84 are prescribed for vehicles manufactured before June 30, 2005, other than vehicles described in subclause (b)(ii)

- the standards set out in the Canadian Standards Association Standard D409-02 are prescribed for vehicles manufactured,

- On or after June 30, 2005, or

- Before June 30, 2005, if the vehicles would have conformed with that Standard if the Standard had been in force at that time.

(2)  Equipment conforming with applicable Standard is prescribed and required for use in all vehicles of the class referred to in subsection (1).

(3) Sections 2 and 3.1 of this Regulation and the emergency exit requirements of section 6.9.1 of Canadian Standards Association Standard D409-M84 and of section 6.4 of Canadian Standards Association Standard D409-02 do not apply to rear-entry accessible taxicabs manufactured on or after January 1, 1986.

(4) The following definitions apply to this section,

“Canadian Standards Association Standard D409-02” means that Standard  and its updates up to and including October, 2004;

“rear-entry accessible taxicab” means an accessible taxicab with a gross vehicle weight rating of not more than 2,900 kilograms that is equipped with a ramp at the rear, a rear door for wheelchair access and two rear side doors for ambulatory access.

9. to be revoked

10.  (1) Inter-city buses that are accessible vehicles designed or modified for the purpose of transporting persons in wheelchairs and equipped with a lift or loading ramp, are exempt from sections 2 and 3.1.

(2) In this section, “inter-city bus” means a bus commonly known as an inter-city motor coach,

(a)	equipped with motive power mounted to the rear of the front axle,

(b)	equipped with air-ride or torsion-bar suspension, and

(c)	equipped with reclining seats and a baggage area that is separate from the passenger cabin.

11.  (1)  An accessible urban transit bus is prescribed as a class of vehicle to which subsection 102 (3)of the Act applies.

(2) to be revoked

(3)  An accessible urban transit bus shall have,

(a) a minimum length of 9.144 metres and a bus chassis as originally provided by the bus manufacturer;

(b) at least two passenger doors on the right side, the front door being at least 762 millimetres wide and the other door being at least 813 millimetres wide;

(c) to be revoked

(d) a system for securing personal mobility devices consisting of devices anchored to the bus or of compartments, or a combination of both, that limit the forward, backward, rotational, lateral and vertical motion of every personal mobility device in the bus at its points of contact with the bus while it is in normal operation, including full throttle acceleration and maximum braking.

(4)  An accessible urban transit bus shall have either,

(a) a power lift to load a passenger mobility device and passenger onto the bus; or

(b) a mostly level floor that, either by design or by the use of a “kneeling” feature, is approximately at the level of a typical urban curbside or bus passenger platform during loading or unloading of passengers and is equipped with a ramp having a slope of no more than 4 to 1 to allow passengers to bridge the gap between the bus and the curbside or platform.

(5) The specification for accessible vehicles set out in sections 2, 3.1, 6, 7 and 8 do not apply to accessible urban transit buses.

(6) “accessible urban transit bus” means a bus that is an accessible vehicle and that is operated as part of a service for which a fare is charged for transporting the public by vehicles operated by or on behalf of a municipality or a local board as defined in the Municipal Affairs Act, or under an agreement between a municipality and a person, firm or corporation.

11. (1) The specifications for accessible vehicles set out in clauses 3(1)(d.1), (d.2) and (f) and in subsections 5(3) and (4) do not apply to accessible vehicles that are school buses within the meaning of section 175 of the Act.

12. to be revoked

13.(1)  Accessible taxicabs that are manufactured by the London Taxis International division of Manganese Bronze Holdings PLC, bear the model name TXII and are equipped with a ramp and two rear side access doors are designated as a class of vehicle that is exempt from section 2, subsections 3.1(2) and (4) and clause 5 (1) (b). O. Reg. 346/02, s. 2.

(2)  Subsection (1) only applies if the vehicles described in that subsection conform to the standards required under the Motor Vehicle Safety Act (Canada) and bear the National Safety Mark referred to in that Act. O. Reg. 346/02, s. 2.